9

[image: image2.png]TOP POLYOFIN TAKE - UPROLLER
—

TOP FILM SUPPLY ROLLER
—
; /

8\,

PEELABLE e
POLYOFIN SHEET 7~

PHOTOGRAPHIC |
EMULSION \

POLYESTER COVER SHEET

SAFETY GUARD
INTERLOCKED WITH DRIVE MOTOR

[ CAMINATOR FEEDTABLE

BOTTOM POLYOFIN

‘SILICON RUBBER
TAE-UPROLLER QOO Rt er ~ PRESSURE DRIVE ROLLERS
‘SILICON RUBBER
HEATED ROLLERS
ADUUSTABLE POSITION

‘GUIDE ROLLER
__— FOREASY THREADING





Photopolymer Dry Film Laminator

The Photopolymer dry film laminators are simple and efficient units for laminating all types of polymer film resists to the clean surface of copper laminates and chemically milled components. The laminator uses accurate controlled temperature and feed to laminate the dry film resist  to the metal surface as well as removing the Polyofin separator sheet. 
Two Models available either 27-22800 (350 mm) or 27-22802 (650 mm).

EASY – TO – USE

	· Exclusive pre-set temperature and speed setting Programs for a variety of applications just by pressing a button on the digital display panel
	· Warm-up time only 10 minutes due to The Infrared heater design which gives better heat distribution and control

	· Adjustable override controls for both temperature and speed to compensate for variations in material or conditions
	· Ready and wait visual and audible signals alert operator to laminator readiness, thus reducing errors

	· Heater roller and pressure roller construction in Silicon rubber, allows roller gap adjustment ensuring correct pressure for panels up to 4mm thick
	· 8 Bit microprocessor digital heat Controller permits precise setting and guarantees fast response times

	· Measure function button displays actual roller temperature during operation.

	· Reverse function button allows easy film clear from the machine. 



	· Instant switch from Centigrade to Fahrenheit
	· Function to allow cold laminating


	MAXIMUM WORKING WIDTH



	27-22800  Film width 305 mm (12”) Film Support Tube 310 mm (12¼”) 

	27-22802  Film width 620 mm (24”) Film Support Tube 620 mm (24½”)


	
	

	TEMPERATURE RANGE

	0 – 160 degrees Centigrade

	LAMINATION SPEEDS
	0 – 1.6  metres/minute variable in 10 stages



	POWER REQUIREMENTS
	220/240 Volts 50/60 Hz single phase 
27-22800
6.0A  Consumption: 1400 watts
27-22802
7.8A  Consumption: 1800 watts 


	SHIPPING DIMENSIONS
	27-22800 MODEL  670 mm (W) x 500 mm (H) x  690 mm(L) 

27-22802 MODEL  670 mm (W) x 500 mm (H) x 1010mm(L)



	SHIPPING WEIGHT
	27-22800 MODEL
      35 kilos
27-22802 MODEL      49 kilos




PHOTOPOLYMER DRY FILM LAMINATOR

OPERATING INSTRUCTIONS

WORK ROOM LIGHTING

Room lighting is most important and Philips TL 65 W 16 strip-lights are suitable for room lighting as their spectrum has no effect on most photoresists. Windows should be covered with non – bleaching yellow foil which cuts out light wavelengths under  450 nm., yellow foil can be obtained from the Kodak company and Plexiglas type  “ YELLOW 303  “ is equally suitable. Processing should be carried out under dust free conditions, the laminated and exposed parts must also be stored in yellow light until used.

THE LAMINATOR

The laminator uses two rolls of Photo-resist, and only two rolls of the same width can be used up to 305 mm in width for the MODEL 305 and 610 mm for the MODEL 610. The dry film photo emulsion is sandwiched between a polyester cover sheet and a peelable Polyofin sheet. The laminator removes the Polyofin cover sheet before the film contacts the heated rollers; these rollers then fuse with heat and pressure the photo emulsion on to the material both sides. On exit the rear cold pressure rollers continue to roll and fix the photo emulsion onto the material.

FILM LOADING AND THREADING

WHEN FILM LOADING USE IN CONJUCTION WITH THE FILM THREADING DIAGRAM ( page 8 & 9 ).

TOP LOADING AND THREADING

1. Remove the feed table and raise the safety guard.

2. Remove the top Polyofin take up roller shaft.

3. Remove the top film supply roller shaft.

4. Fit one of the spare cardboard tubes between the two core supports of the top Polyofin take up roller shaft.

5. Fit one of the DRY FILM rolls between the two core supports of the top supply roller shaft.

6. Replace the top supply roller shaft with roll of film onto the machine, the round bearing end fits in the side with the steel spring clip first and the hexagon end fits into the hexagon film tension socket. Ensure the film roll is loaded the correct way, film dropping off the back of the roll.

7. Replace the top Polyofin take up roller shaft.

8. Adjust the lateral position of both dry film roll and cardboard tube to align with each other.

9. Remove the end of the top Polyofin film and thread up the top section as per the threading diagram on page 8 and stick the edge of the Polyofin film to the top take up roller tube with masking tape across the tube width.  If however the film is sticking when running laminator, re-thread as per diagram on page 9
10. Roll both rolls together to allow excess film to feed behind the guard and below the centre line of the two heat rollers.

BOTTOM LOADING  AND THREADING

1. Remove the bottom Polyofin take up roller shaft.

2. Remove the bottom film supply roller shaft.

3. Fit the other spare cardboard tube between the two core supports of the bottom Polyofin take up roller shaft.

4. Fit the second roll of DRY FILM between the two core supports of the bottom supply roller shaft.

5. Replace the bottom supply roller shaft with roll of film onto the machine, the round bearing end fits in the side with the steel spring clip first and the hexagon end fits into the hexagon film tension socket. Ensure the film roll is loaded the correct way, film dropping off the front of the roll.

6. Replace the bottom Polyofin take up roller shaft.

7. Adjust the lateral position of the bottom DRY FILM roll to align with the top DRY FILM roll, measure if necessary from the side plate to the side of each roll to ensure accurate alignment. If these rolls are miss-aligned then a build up of emulsion from the dry film will be come attached to the silicon front heat and rear pressure rollers causing problems.

8. Adjust the lateral position of the bottom Polyofin cardboard tube to align with the dry film roll.

9. Remove the end of the bottom Polyofin film and tread up the bottom section as per the threading diagram, the bottom guide roller can be easily pulled forward via the curved slots to allow easy threading. Stick the edge of the Polyofin film to the bottom take up roller tube with masking tape across the tube width.

10. Roll both rolls together to allow excess film to feed upwards above the heat rolls and feed up behind the safety guard and back over so to remain in position.

11. Lift the pressure release lever on the right side of the laminator until vertical, this separates the rollers, then feed a scrap piece of board between the rollers forcing both films through the rollers.

12. Lower the pressure release lever and secure in working position. ( This position is adjustable for different thickness of panels giving  increase roller pressure the lower the handle ).

13. Lower the safety guard. ( the guard has two switch interlocks both ends which inhibit the motor rotation )

14. Switch the machine on with the ON/OFF rocker switch at the rear of the laminator adjacent to the mains input cable entry, the temperature defaults to zero degrees centigrade, adjust the motor speed on the front panel to No 4 and switch the motor on with “ RUN”, allow to feed so that the scrap card exits fully from the rear of the laminator and then stop the motor with “ STOP”. If during this operation film jam problems occur then the “REVERSE” button  can be pressed to eject the film and allow this operation to be repeated.

15. Re-position the bottom guide roller via the curved slots to the top running position.

16. Run off about 1 metre of film to allow creases and misalignment to correct.

17. If both films entry point into the heat rollers are aligned then the laminator is filmed, if not check the positions of both dry film rolls from the end plates and re-adjust if necessary. Replace the machine table and align the work feed stop to within 5 mm of the inside edge of the dry film.

The laminator is now threaded with film and ready to run, it is advisable to check with the manufacturer the recommended laminating temperature of the dry film, the feed speed and the exit temperature required. As with some Dry Films the peelable POLYOFIN sheet can have a low melting temperature. The guide roller adjacent to the machine guard which is situated above the top heat roller can get hot. When the machine is switched off this POLYOFIN film can stick to the cooling guide roller. When next the machine  is switched on and the drive motor activated this POLYOFIN film can break, if this happens then use the alternative treading diagram on page 5.

MACHINE OPERATION

The machine is very simple to use after threading. Switch the laminator on with the rear rocker mains switch, the laminator defaults to room temperature setting, motor off and degrees Centigrade. Refer to the film temperature and feed rate speed specification from the dry film data sheet, select one of the four pre-set programs “38” “75” “125” “250” on the front panel, the temperature setting can be modified by using the up and down temperature arrowhead keys to select the required temperature recommended by the Dry Film Manufacturer, similarly using the up and down arrowhead keys select the required film feed speed. 

The laminator now will start to heat up to the set temperature and this takes about 10 – 12 minutes dependant on the temperature selected, up to a maximum high of 160 degrees centigrade. When the temperature of the heat rollers reach 5 degrees lower than the selected temperature the “READY” sign will be displayed on the front panel, and when the temperature rises 5 degrees above the set temperature the  “READY” sign will flash. This over shoot will happen on initial warm up and it is best to leave the laminator a further 3 – 4 minutes to stabilise the temperature before use. At any time the “MEAS” button on the front panel can be pressed to display the actual roller surface temperature of the machine. To laminate the panels now press  “RUN” the motor will slowly increase speed to the selected speed number and lamination can commence by feeding the panels against the side fence on the table and into the pinch grip of the front rollers. As soon as the first panel has been through the front heat rollers then introduce the second panel so as not to waste dry film and continue until all panels have been laminated, as the panels exit they can be stacked in a zigzag fashion and separated later with a sharp knife, the  “ STOP” button then switches the motor off. The “FAN” button is used to switch a cooling fan on/off, this is situated between the front heat rollers and the rear pull rollers, it is only operated when the exit temperature of the laminated panels is above the recommended exit temperature of the Dry Film manufacturers specification, usually needed when laminating metal plates which can retain heat longer than printed circuit boards.

If the machine is left switched on with a high temperature selected and without running for 90 minutes then the microprocessor will shut the temperature down and remain in a standby mode until required for use, the digital display will show  “st by”, operation of one of the buttons will re-instate the program settings used.

ROLLER PRESSURE SETTINGS
	BOTTOM POSITION
	Zero to 1/16th inch thick panels

	NEXT UP POSITION
	1/16th inch thick panels to 3/32nd inch thick panels

	NEXT UP POSITION
	3/32nd inch thick panels to 1/8th inch thick panels

	NEXT UP POSITION
	Depending on panel thickness and pressure required

	NEXT UP POSITION
	Depending on panel thickness and pressure required

	NEXT UP POSITION
	Depending on panel thickness and pressure required


Top position is a park position, so that the rollers do not produce flat spots when not in use, and machine should always be left in this position when you have finished laminating.
SPEED SETTINGS
	PROGRAM No
	SPEED

	9
	1.50 metres / min.

	8
	1.35 metres / min.

	7
	1.15 metres / min.

	6
	1.00 metres / min.

	5
	0.85 metres / min.

	4
	0.65 metres / min.

	3
	0.50 metres / min.

	2
	0.35 metres / min.

	1
	0.15 metres / min.


The above speeds settings are approximate and ‘UNDER’ load.  
PRE-PROGRAM SETTINGS
The laminator has 4 pre-programmed settings to cover most of the common films used.

	PROGRAM No
	TEMPERATURE
	SPEED No.
	SPEED

	38
	120 degrees Centigrade
	8
	1.4 metres / min.

	75
	115 degrees Centigrade
	6
	1.0 metres / min.

	125
	110 degrees Centigrade
	5
	0.9 metres / min.

	250
	105 degrees Centigrade
	3
	0.5 metre / Min


PANEL PREPARATION

Sufficient adhesion, necessary in the subsequent etching and electrolytic processes can only be ensured if the panel material surface is absolutely free of grease and oxides as well as any moisture or traces left by mechanical/chemical cleaning. This cleaning may be carried out by hand or machine with suitable advice from the supplier of DRY FILM.
STORAGE

The waiting periods before panel preparation and lamination should not exceed 60 minutes, otherwise there is a chance that the metal surface may become oxidised and cause loss of adhesion. After laminating, the panels must be allowed to stand at room temperature for 20 minutes before exposure to ensure the cure of the dry film emulsion.

The resists of most manufacturers require that the dry films be stored at a temperature of less than 26 degrees centigrade but preferably between 15 – 20 degrees centigrade and at a relative humidity of  40% - 60 % to ensure full shelf life.

[image: image3.jpg][image: image4.jpg]PEELABLE

. TOP POLYOFIN
POLYOFIN SHEET ~

- TAKE - UP ROLLER

P

TOP FILM SUPPLY ROLLER
PHOTOGRAPHIC 0» S

EMULSION e
-
POLYESTER COVER SHEET
SAFETY GUARD i Q o = GUIDE ROLLERS
INTERLOCKED WITH DRIVE MOTOR L =

. LAMINATORFEEDTABLE |
BOTTOM POLYOFIN
TAKE - UP ROLLER BOTTOM FILM

, SUPPLY ROLLER

" SILICON RUBBER
PRESSURE DRIVE ROLLERS

! 'SILICON RUBBER
| HEATED ROLLERS

SUDE RoER SN ALTERNATIVE
.- FOR EASY THREADING
THREADING DIAGRAM




[image: image5.jpg]03 _Lavd|

] pnoy |31V NIISIATY
[oN_naAva] Sm | e v
R T v
N N aTeag @I 2 oot Gead/ns| v
W
7 v

—||e [w]in] 0|~ |0l [ 2= | M2 (X2





[image: image6.jpg]


[image: image7.jpg]



[image: image1.png]



Part Nos.: 


27-22800 -12”  


27-22802 -24” Laminators








總代理: 吉 爾 泰 國 際 有 限 公 司


               Gigatech International Co., Ltd.





110台北市信義區忠孝東路五段482號9樓之7


Tel:	886-2-27279958


Fax:	886-2-27279658


Email: giga1688@ms78.hinet.net





7





8











G:\Data\Nicola\27-22800  Laminator Instructions Gigatech (no MEGA website).doc

